COMSATS University Islamabad, Lahore Campus

Department of Computer Engineering

Assignment 1 – FALL 2025

| Course Title: | Digital System Design |  |  |  | Course Code: | CPE 344 | Credit Hours: | 4(3,1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Course Instructor: | Dr. Abbas Javed |  |  |  | Program Name: | BCE |  |  |
| Semester: | 6th | Batch: | FA23 | Section: | A & B | Date Given: |  |  |
| Submission Date: | March 31, 2026 |  |  |  | Maximum Marks: |  | 40 |  |
| Name: |  |  |  |  | Registration Number: |  |  |  |
| Important Instructions / Guidelines: - Do your own work, PLAGARISM will be graded as ZERO. - No late submission. |  |  |  |  |  |  |  |  |

Question 1:						[CLO1-PLO3-C5]		[12+4+2+2 Marks]
Design a finite state machine (FSM) for an automatic hand sanitizer dispenser where input X represents a hand detection sensor (1 when a hand is present, 0 otherwise) and output Y controls the sanitizer pump (1 = dispense, 0 = off); whenever a hand is detected (i.e., X changes from 0 to 1), the system should activate the pump and keep it ON for exactly four clock cycles to dispense a fixed amount of sanitizer, after which the pump must turn OFF regardless of whether the hand is still present, and the system should not activate again until the hand is removed (X returns to 0) and then detected again, and using the controller design methodology, construct the FSM, define and encode the states, derive the state transition table, and finally obtain the controller truth table (stop after the truth table).

Question 2:						[CLO1-PLO3-C5]		[5+5+5+5 Marks]
A smart home control panel can display one of four modes: lighting, temperature, security, or energy usage, controlled by two signals s1 and s0 (00 displays lighting, 01 temperature, 10 security, and 11 energy usage—assume s1 and s0 control an N-bit multiplexer that selects the corresponding data). Pressing a button B (which sets B = 1) cycles the display to the next mode; for example, if the current mode is energy usage, the next mode should be lighting. Create a state diagram for an FSM describing this sequencing behavior, with input B and outputs s1 and s0. Ensure that the system advances only one mode per button press regardless of how long the button is held, meaning the FSM must wait for the button to be released before allowing another transition. Use short but descriptive names for each state, and assume the initial state displays lighting. Design the controller by using state registers and logic gates.