COMSATS University Islamabad, Lahore Campus

Department of Computer Engineering

Assignment 2 – Spring 2026

| Course Title: | Digital System Design |  |  |  | Course Code: | CPE 344 | Credit Hours: | 4(3,1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Course Instructor: | Dr. Abbas Javed |  |  |  | Program Name: | BCE |  |  |
| Semester: | 6th | Batch: | FA23 | Section: | A & B | Date Given: |  |  |
| Submission Date: | April 24, 2026 |  |  |  | Maximum Marks: |  | 20 |  |
| Name: |  |  |  |  | Registration Number: |  |  |  |
| Important Instructions / Guidelines: - Do your own work, PLAGARISM will be graded as ZERO. - No late submission. |  |  |  |  |  |  |  |  |

Question 1:						[CLO1-PLO3-C5]		                [10 Marks]
Design a 4-bit brightness control system for an LED panel that uses a counter to adjust intensity levels. The system has four control inputs: inc to increase brightness, dec to decrease brightness, reset to synchronously set the brightness level to minimum (0000), and max to synchronously set it to maximum (1111). If more than one control input is active at the same time, the brightness level should remain unchanged. Use a 4-bit parallel load register as the main building block to implement the design, along with any additional logic required.

Question 2:						[CLO1-PLO3-C5]		                  [10Marks]
Design a digital grading system for an exam evaluation platform that determines whether a student’s estimated score is within an acceptable margin of the actual score. The system has an 8-bit input E representing the estimated score and an 8-bit input A representing the actual score assigned by the examiner. The system should output a single bit C, where C = 1 if the estimated score is within ±5 marks of the actual score, and C = 0 otherwise. For example, if E = 70 and A = 75, the output is 1, but if E = 69 and A = 75, the output is 0. Design this system using 8-bit magnitude comparators and any additional logic or components as required.