COMSATS University Islamabad, Lahore Campus

Department of Computer Engineering

Assignment 3 – Spring 2026

| Course Title: | Digital System Design |  |  |  | Course Code: | CPE 344 | Credit Hours: | 4(3,1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Course Instructor: | Dr. Abbas Javed |  |  |  | Program Name: | BCE |  |  |
| Semester: | 6th | Batch: | FA23 | Section: | A & B | Date Given: |  |  |
| Submission Date: | April 30, 2026 |  |  |  | Maximum Marks: |  | 20 |  |
| Name: |  |  |  |  | Registration Number: |  |  |  |
| Important Instructions / Guidelines: - Do your own work, PLAGARISM will be graded as ZERO. - No late submission. |  |  |  |  |  |  |  |  |

Question 1:						[CLO2-PLO3-C5]		                [10 Marks]
Design a smart controller for indoor room light and temperature control using RTL design process. Smart controller should turn on the light and inverter air conditioner when an occupant enters the room and turn off the lights/air conditioner when occupant leaves the room after the specific time (you are free to choose any time period). Smart controller should maintain the comfortable room temperature by setting the required temperature when room is occupied. Choose suitable sensors for occupancy detection. Using RTL design process, design the smart controller for summers.

You are required to complete the following steps:

Step 1: Create High Level State Machine (HLSM)

Step 2: Create a datapath.

Step 3: Connect the datapath to a controller.

Step 4: Derive the controller’s FSM.

Question 2:						[CLO2-PLO3-C5]		                  [10Marks]
Design a system for a hospital patient monitoring unit that repeatedly determines and outputs the lowest heart rate recorded among 64 patients stored in a register file A, where each entry is a 32-bit unsigned value representing a patient’s heart rate measurement. The system should scan all patient records, compare values to find the minimum heart rate, and store the result in an output register min_hr for medical staff to review. Assume appropriate control signals such as go to start the operation and done to indicate completion, and design the system using the RTL methodology by first modeling the behavior as an HLSM and then converting it into a controller and datapath.